# Firearm Written Inspection Checklist

Instructions:

Pictures are required where noted and should be taken with the iPad – Visually document as much as practical.

If you run out of space to make notes - Use the final page a reference Section and Letter in your comments -- Example – 1B -  firearm is covered in extreme rust and pitting – new parts and significant efforts will be required to make it operable.

| Work Order # | _____________________ | Date Inspected: | _____________________ |
| --- | --- | --- | --- |
| Customer Name: | _____________________ | Inspector Name: | _____________________ |
| Make: | _____________________ | Model: | _____________________ |
| Serial #: | _____________________ | Caliber/Gauge: | _____________________ |
| Action Type: | _____________________ | Stated Problem: | _____________________ |

| ## Intake and Safety |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Physically verify firearm is unloaded; remove magazine or source of ammunition. |  |  |  |  |
| - Observe and write down the overall condition in comments / notes. NOTE ANY rust, dirt, damage, or missing parts. |  |  |  |  |
| - (Pics) Take general photographs of the firearm from all angles. (Pics) Take photos of included accessories. |  |  |  |  |

| ## External Inspection (Assembled) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - (Pics) Open action and visually inspect chamber, bore path, and feed area. |  |  |  |  |
| - (Pics) Inspect frame/receiver for cracks, deformation, stripped screws, or impact damage. |  |  |  |  |
| - (Pics) Inspect barrel exterior, muzzle, crown, and visible threads for damage. |  |  |  |  |
| - (Pics) Inspect grips, stock, handguard, or forend for looseness or cracks. |  |  |  |  |
| - Inspect sights, optic mounts, rails, and fasteners for security. |  |  |  |  |
| - Inspect controls (safety, mag release, etc.) |  |  |  |  |
| - (Pics) Inspect magazines or feeding devices for dents, cracks, spread lips. |  |  |  |  |
| - Perform a 20 in /lb torque test on all visible screws – Note any failures |  |  |  |  |
| - Perform a spring test by hand on all available springs prior to disassembly. – Note any weak spring areas |  |  |  |  |

| ## Function Check Before Disassembly |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Cycle action using dummy rounds slowly and quickly; note drag, binding, or abnormal resistance. |  |  |  |  |
| - Check trigger movement, reset, hammer, striker, and sear engagement. |  |  |  |  |
| - Check manual safety, decocker, transfer bar, and firing block function. |  |  |  |  |
| - Check magazine insertion, retention, and release. |  |  |  |  |
| - Check lockup, bolt or slide travel, and return to battery. |  |  |  |  |
| - Performs gas system air test, if applicable. |  |  |  |  |

| ## Disassembly |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Measure and record firing pin protrusion |  |  |  |  |
| - Field strip or detail strip as needed to perform a quality inspection |  |  |  |  |
| - (Pics) Lay out parts in order and visually observe any non-factory or altered components. |  |  |  |  |
| - Inspect under magnification the, bolt or slide face, breech area, and action components. |  |  |  |  |

| ## Internal Inspection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - (Pics) Perform a borescope inspection of the bore and chamber(s) Inspect bore and chamber for fouling, pitting, bulges, rings, or rough areas. |  |  |  |  |
| - Measure firing pin protrusion Finger test firing pin spring tension and make sure the spring returns the pin into position. |  |  |  |  |
| - Check headspace. - Remove extractor/ ejectors if required. Method 1 – Gauge Method 2 - Shims |  |  |  |  |
| - Under magnification inspect all internals parts for cracks, peening, or abnormal wear. |  |  |  |  |
| - Inspect and spring test extractor, ejector, firing pin, striker, and related springs. |  |  |  |  |
| - Measure Firing Pin OAL – Compare to factory specs if available |  |  |  |  |
| - Inspect the locking surfaces, lugs, rails, and bearing surfaces for excessive wear, rust, or uneven surfaces. |  |  |  |  |
| - Inspect fire control parts for unsafe wear, breakage, or prior alteration. |  |  |  |  |
| - Inspect recoil, action, hammer, magazine, or return springs. |  |  |  |  |
| - Count the total number of disassembled parts |  |  |  |  |

| - Gas Systems where applicable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Pull a manufacturer diagram to ensure all gas system parts are present |  |  |  |  |
| - Borescope the gas port and take pictures if there is any wear |  |  |  |  |
| - Measure and record the gas port diameter. Compare against manufacturer specifications if available. |  |  |  |  |
| - Air test all applicable gas component individually watching for flow issues |  |  |  |  |
| - Inspect piston for uneven wear or damage |  |  |  |  |
| - Check alignment of the gas block to the receiver - If applicable |  |  |  |  |
| - Disassemble any gas blocks that allow it. Check for carbon deposit buildup and wear. |  |  |  |  |
| - Inspect O-rings and piston rings for wear, tension, and proper seating. Recommend replacement if any damage or significant wear is found. |  |  |  |  |

| - Cleaning – Often Bundled with Inspection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Perform deep cleaning, if included in the work order. |  |  |  |  |
| - Verify the ultrasonic is clean and warmed up. |  |  |  |  |
| - Clean and oil all components thoroughly. Oil of components may require full dunks. |  |  |  |  |
| - Perform oil bath (minimum 2 hours). |  |  |  |  |

| ## Reassembly |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Count the total number of parts and confirm all parts are present. |  |  |  |  |
| - Reassemble in correct order with proper lubrication. |  |  |  |  |
| - Verify all pins, screws, caps, and retainers are seated. |  |  |  |  |
| - Set torque to 30 in/lb per standard screw(s). Adjust higher for action screws, if needed, to ensure proper seating. |  |  |  |  |
| - Assess all screws for stress, vibration, or failure risk. Apply Loctite, where necessary. |  |  |  |  |
| - Manually conduct a post-assembly function check using dummy rounds. Observe and note feed path, ejection, extraction, and spring tensions. |  |  |  |  |
| - Check and verify all safety systems again after assembly. |  |  |  |  |

| ## Live Fire/Range Evaluation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Write down the ammunition brand and type used for testing. |  |  |  |  |
| - Follow test procedure below based on the type of firearm: |  |  |  |  |
| - Observe and evaluate feeding, ignition, extraction, ejection, lockup, and accuracy. |  |  |  |  |
| - Track results by magazine or feeding device. |  |  |  |  |
| - Inspect fired cases for pressure signs, rim damage, bulges, or unusual marks. |  |  |  |  |
| - Save 1-3 fired cases and examine them under magnification in the shop. The cases are to be returned to the client with their firearm. Record any data gathered from fired cases. |  |  |  |  |
| TEST PROCEDURES: Test firing procedures vary based on the type of gun. Our standard test for semi-automatics is 1 round, 3 rounds and 6 rounds. The 1 round ensure bolt hold open and test for feeding / extraction issue / sticky chamber etc. 3 rounds further test to ensure that we didn't create a fully automatic weapon and to further examine the action. 6 rounds is a rapid fire stress test. In a revolver we usually fire one cylinder via single action and one cylinder via double action to ensure reliability in all spring configurations. Shotguns we are limited to 2 shells rapid fire. Most shotgun tests are a rapid cycle test followed by 1 shot, followed by 2 shots slow and 2 shots fast all with factory trap loads. Bolt guns, single shots and most others get 3-5 rounds. |  |  |  |  |

| ## Final Disposition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inspection Item | Pass | Fail | N/A | Comments/Measured Values |
| - Create a shared album on the iPad and share with client via link |  |  |  |  |
| - Firearm passes inspection/testing. |  |  |  |  |
| - Firearm passes with recommendations. |  |  |  |  |
| - Repair required before return. |  |  |  |  |
| - Unsafe condition found. |  |  |  |  |

## Notes

Inspector Signature: __________________________

Date: __________________________